# CS 385 – FINAL REPORT

RACHAEL GRABOWSKI & MICHAEL LANG

# INSTRUCTION SET

| Instruction | OP Code | Instruction | OP Code |
| --- | --- | --- | --- |
| add | 0000 | sw | 0110 |
| sub | 0001 | slt | 0111 |
| and | 0010 | beq | 1000 |
| or | 0011 | bne | 1001 |
| addi | 0100 | sll | 1010 |
| lw | 0101 | srl | 1011 |

Our implementation of a 16-bit mini pipelined MIPS machine (using gate modeling and behavioral modeling in Verilog) includes the following instructions: add, sub, and, or, addi, lw, sw, slt, beq, bne, sll, srl (see the table above for corresponding OP codes).

| R-Format |  |  |  |  |
| --- | --- | --- | --- | --- |
| OP | RS | RT | RD | Unused |
| 4 | 2 | 2 | 2 | 6 |

| I-Format |  |  |  |
| --- | --- | --- | --- |
| OP | RS | RT | Address/Value |
| 4 | 2 | 2 | 8 |

# Major Component Descriptions

## Instruction Memory

The instruction memory is represented by our input.hex file (see “input.hex” below)

## ALU

The ALU is a 16-bit ALU comprised of four 4-bit ALUs. It is capable of detecting overflow and zero.

## Register File

Register r0 is static 0 (it cannot change or be changed). The register file is 4-word by 16-bits-per-word.

## Control Unit

The mainCtrl unit takes IFID_InstrMem[15:12] as input, and outputs a control signal to the IDEX register. The control options are: RegDst, BEQ, BNE, MemToReg, MemWrite, ALUsrc, RegWrite, SLL, SRL, and ALUctl.

## input.hex

The input.hex file is how we loaded our code into our program, instead of hard-coding our testing instructions. This made it very easy to swap between different sets of instructions for testing at various points in the project. MIPS ASM instructions were converted into binary, and those binary instructions were then converted into hex instructions. Those hex instructions were then read by the CPU module into instruction memory.

# DIAGRAMS

# VERILOG SOURGE CODE

## module alu_1_bit(result, c_out, a, b, less, c_in, op);

input a, b;

input less;

input c_in;

input [2:0] op;

output result, c_out;

wire m, n, p, q, r;

not g1(m, b);

mux_2_to_1  mux1(n, b, m, op[2]);

and g2(p, a, n);

or  g3(q, a, n);

full_adder  fa(r, c_out, a, n, c_in);

mux_4_to_1  mux2(result, p, q, r, less, op[1:0]);

endmodule

## module alu_1_bit_msb(result, overflow, set, c_out, a, b, less, c_in, op);

input a, b;

input less;

input c_in;

input [2:0] op;

output result, overflow, set, c_out;

wire m, n, p, q;

not g1(m, b);

mux_2_to_1  mux1(n, b, m, op[2]);

and g2(p, a, n);

or  g3(q, a, n);

full_adder  fa(set, c_out, a, n, c_in);

mux_4_to_1  mux2(result, p, q, set, less, op[1:0]);

xor g4(overflow, c_out, c_in);

endmodule

## module alu_4_bit(result, zero, c_out, a, b, less, c_in, op);

input [3:0] a;

input [3:0] b;

input less;

input c_in;

input [2:0] op;

output [3:0] result;

output zero, c_out;

wire p, q, r, s, t;

alu_1_bit alu_0(result[0], p, a[0], b[0], less, c_in, op),

alu_1(result[1], q, a[1], b[1], 0, p, op),

alu_2(result[2], r, a[2], b[2], 0, q, op),

alu_3(result[3], c_out, a[3], b[3], 0, r, op);

or  g1(s, result[0], result[1]),

g2(t, result[2], result[3]);

nor g3(zero, s, t);

endmodule

## module alu_4_bit_last(result, overflow, set, zero, c_out, a, b, c_in, op);

input [3:0] a;

input [3:0] b;

input c_in;

input [2:0] op;

output [3:0] result;

output overflow, set, zero, c_out;

wire p, q, r, s, t;

alu_1_bit alu_0(result[0], p, a[0], b[0], 0, c_in, op),

alu_1(result[1], q, a[1], b[1], 0, p, op),

alu_2(result[2], r, a[2], b[2], 0, q, op);

alu_1_bit_msb alu_3(result[3], overflow, set, c_out, a[3], b[3], 0, r, op);

or  g1(s, result[0], result[1]),

g2(t, result[2], result[3]);

nor g3(zero, s, t);

endmodule

## module alu_16_bit(result, overflow, zero, c_out, a, b, op);

input [15:0] a;

input [15:0] b;

input [2:0] op;

output [15:0] result;

output overflow, zero, c_out;

wire p, q, r, s, t, u, v, w, x;

wire less;

alu_4_bit alu_0(result[3:0], s, p, a[3:0], b[3:0], less, op[2], op),

alu_1(result[7:4], t, q, a[7:4], b[7:4], 0, p, op),

alu_2(result[11:8], u, r, a[11:8], b[11:8], 0, q, op);

alu_4_bit_last alu_3(result[15:12], overflow, less, v, c_out, a[15:12], b[15:12], r, op);

and  g1(w, s, t),

g2(x, u, v),

g3(zero, w, x);

endmodule

## module d_flip_flop(D,CLK,Q);

input D,CLK;

output Q;

wire CLK1, Y;

not  not1 (CLK1,CLK);

d_latch D1(D,CLK, Y),

D2(Y,CLK1,Q);

endmodule

## module d_latch(D,C,Q);

input D,C;

output Q;

wire x,y,D1,Q1;

nand nand1 (x,D, C),

nand2 (y,D1,C),

nand3 (Q,x,Q1),

nand4 (Q1,y,Q);

not  not1  (D1,D);

endmodule

## module decoder_2_to_4(out, sel);

input [1:0] sel;

output [3:0] out;

wire not_sel0, not_sel1;

not n0(not_sel0, sel[0]),

n1(not_sel1, sel[1]);

and a0(out[0], not_sel0, not_sel1),

a1(out[1], sel[0], not_sel1),

a2(out[2], not_sel0, sel[1]),

a3(out[3], sel[0], sel[1]);

endmodule

## module half_adder(sum, c_out, a, b);

input a, b;

output sum, c_out;

xor g1 (sum, a, b);

and g2 (c_out, a, b);

endmodule

## module full_adder(sum, c_out, a, b, c_in);

input a, b, c_in;

output sum, c_out;

wire p, q, r;

half_adder ha1(p, q, a, b),

ha2(sum, r, p, c_in);

or g1(c_out, r, q);

endmodule

## module mux_2_to_1(x, a, b, sel);

input a, b, sel;

output x;

wire p, q, r;

not g1 (p, sel);

and g2 (q, a, p),

g3 (r, b, sel);

or  g4 (x, q, r);

endmodule

## module mux_4_to_1(x, a, b, c, d, sel);

input a, b, c, d;

input [1:0] sel;

output x;

wire p, q;

mux_2_to_1 mux1(p, a, b, sel[0]),

mux2(q, c, d, sel[0]),

mux3(x, p, q, sel[1]);

endmodule

## module mux_4_to_2(x, a, b, sel);

input [1:0] a, b;

input sel;

output [1:0] x;

mux_2_to_1 mux1(x[0], a[0], b[0], sel),

mux2(x[1], a[1], b[1], sel);

endmodule

## module mux_32_to_16(x, a, b, sel);

input [15:0] a, b;

input sel;

output [15:0] x;

mux_2_to_1 m0(x[0], a[0], b[0], sel),

m1(x[1], a[1], b[1], sel),

m2(x[2], a[2], b[2], sel),

m3(x[3], a[3], b[3], sel),

m4(x[4], a[4], b[4], sel),

m5(x[5], a[5], b[5], sel),

m6(x[6], a[6], b[6], sel),

m7(x[7], a[7], b[7], sel),

m8(x[8], a[8], b[8], sel),

m9(x[9], a[9], b[9], sel),

m10(x[10], a[10], b[10], sel),

m11(x[11], a[11], b[11], sel),

m12(x[12], a[12], b[12], sel),

m13(x[13], a[13], b[13], sel),

m14(x[14], a[14], b[14], sel),

m15(x[15], a[15], b[15], sel);

endmodule

## module mux_64_to_16(x, a, b, c, d, sel);

input [15:0] a, b, c, d;

input [1:0] sel;

output [15:0] x;

mux_4_to_1 m0(x[0], a[0], b[0], c[0], d[0], sel),

m1(x[1], a[1], b[1], c[1], d[1], sel),

m2(x[2], a[2], b[2], c[2], d[2], sel),

m3(x[3], a[3], b[3], c[3], d[3], sel),

m4(x[4], a[4], b[4], c[4], d[4], sel),

m5(x[5], a[5], b[5], c[5], d[5], sel),

m6(x[6], a[6], b[6], c[6], d[6], sel),

m7(x[7], a[7], b[7], c[7], d[7], sel),

m8(x[8], a[8], b[8], c[8], d[8], sel),

m9(x[9], a[9], b[9], c[9], d[9], sel),

m10(x[10], a[10], b[10], c[10], d[10], sel),

m11(x[11], a[11], b[11], c[11], d[11], sel),

m12(x[12], a[12], b[12], c[12], d[12], sel),

m13(x[13], a[13], b[13], c[13], d[13], sel),

m14(x[14], a[14], b[14], c[14], d[14], sel),

m15(x[15], a[15], b[15], c[15], d[15], sel);

endmodule

module register_16_bit(out, in, clk);

input [15:0] in;

input clk;

output [15:0] out;

d_flip_flop d0(in[0], clk, out[0]),

d1(in[1], clk, out[1]),

d2(in[2], clk, out[2]),

d3(in[3], clk, out[3]),

d4(in[4], clk, out[4]),

d5(in[5], clk, out[5]),

d6(in[6], clk, out[6]),

d7(in[7], clk, out[7]),

d8(in[8], clk, out[8]),

d9(in[9], clk, out[9]),

d10(in[10], clk, out[10]),

d11(in[11], clk, out[11]),

d12(in[12], clk, out[12]),

d13(in[13], clk, out[13]),

d14(in[14], clk, out[14]),

d15(in[15], clk, out[15]);

endmodule

module register_file_4_by_16(read_data1, read_data2, read_reg1, read_reg2, write_reg, write_data, reg_write, clk);

input [1:0] read_reg1, read_reg2, write_reg;

input [15:0] write_data;

input reg_write, clk;

output [15:0] read_data1, read_data2;

wire [3:0] w, c;

wire p;

wire [15:0] q1, q2, q3;

decoder_2_to_4 dec(w, write_reg);

and g1(p, reg_write, clk),

g2(c[1], p, w[1]),

g3(c[2], p, w[2]),

g4(c[3], p, w[3]);

register_16_bit r1(q1, write_data, c[1]),

r2(q2, write_data, c[2]),

r3(q3, write_data, c[3]);

mux_64_to_16 m1(read_data1, 0, q1, q2, q3, read_reg1),

m2(read_data2, 0, q1, q2, q3, read_reg2);

endmodule

## module shift_left_logical(out, in, shift_amount);

input [15:0] in;

input [3:0] shift_amount;

output [15:0] out;

wire [15:0] p, q, r;

mux_32_to_16 mux1(p, in[15:0], {in[7:0],8'b0}, shift_amount[3]),

mux2(q, p, {p[11:0],4'b0}, shift_amount[2]),

mux3(r, q, {q[13:0],2'b0}, shift_amount[1]),

mux4(out, r, {r[14:0],1'b0}, shift_amount[0]);

endmodule

## module shift_right_logical(out, in, shift_amount);

input [15:0] in;

input [3:0] shift_amount;

output [15:0] out;

wire [15:0] p, q, r;

mux_32_to_16 mux1(p, in[15:0], {8'b0,in[15:8]}, shift_amount[3]),

mux2(q, p, {4'b0,p[15:4]}, shift_amount[2]),

mux3(r, q, {2'b0,q[15:2]}, shift_amount[1]),

mux4(out, r, {1'b0,r[15:1]}, shift_amount[0]);

endmodule

## module CPU (clock, PC, IFID_InstrReg, IDEX_InstrReg, EXMEM_InstrReg, MEMWB_InstrReg, WriteData);

input clock;

output [15:0] PC, WriteData, IFID_InstrReg, IDEX_InstrReg, EXMEM_InstrReg, MEMWB_InstrReg;

// IF

wire [15:0] NextPC, PCplus1;

reg [15:0] PC, IFID_InstrReg;

reg [15:0] InstrMem[0:511];

alu_16_bit fetch(PCplus1, unused1, unused2, unused3, PC, 1, 3'b010);

mux_32_to_16 mux1(NextPC, PCplus1, EXMEM_SignExtend, PCsrc);

// Read hex data from text file into InstrMem

initial begin

$readmemh("input4.hex", InstrMem);

DataMem[0] = 16'h400;

DataMem[1] = 16'hfad;

end

// ID

reg [15:0] IDEX_InstrReg;

wire [11:0] control;

reg IDEX_RegDst, IDEX_BEQ, IDEX_BNE, IDEX_MemToReg, IDEX_MemWrite, IDEX_ALUsrc, IDEX_RegWrite, IDEX_SLL, IDEX_SRL;

reg [2:0] IDEX_ALUctl;

wire [15:0] ReadData1, ReadData2, SignExtend, WriteData;

reg [15:0] IDEX_ReadData1, IDEX_ReadData2, IDEX_SignExtend;

reg [1:0] IDEX_rt, IDEX_rd;

reg [3:0] IDEX_shamt;

register_file_4_by_16 regFile(ReadData1, ReadData2, IFID_InstrReg[11:10], IFID_InstrReg[9:8], MEMWB_WriteReg, WriteData, MEMWB_RegWrite, clock);

main_control mainCtrl(IFID_InstrReg[15:12], control);

assign SignExtend = {{8{IFID_InstrReg[7]}},IFID_InstrReg[7:0]}; // sign extension

// EX

wire [15:0] B, ALUout, ShiftLOut, ShiftROut, Shifted;

wire [1:0] WriteReg;

wire Zero, ShiftToReg;

reg EXMEM_RegWrite, EXMEM_MemToReg;

reg EXMEM_MemWrite, EXMEM_BEQ, EXMEM_BNE, EXMEM_ShiftToReg;

reg EXMEM_Zero;

reg [15:0] EXMEM_SignExtend, EXMEM_ALUout, EXMEM_ReadData2, EXMEM_Shifted;

reg [15:0] EXMEM_InstrReg;

reg [1:0] EXMEM_WriteReg;

alu_16_bit ex(ALUout, unused4, Zero, unused5, IDEX_ReadData1, B, IDEX_ALUctl);

mux_4_to_2 mux2(WriteReg, IDEX_rt, IDEX_rd, IDEX_RegDst);

mux_32_to_16 mux3(B, IDEX_ReadData2, IDEX_SignExtend, IDEX_ALUsrc);

shift_left_logical sll(ShiftLOut, IDEX_ReadData2, IDEX_shamt);

shift_right_logical srl(ShiftROut, IDEX_ReadData2, IDEX_shamt);

mux_32_to_16 mux4(Shifted, ShiftLOut, ShiftROut, IDEX_SRL);

or g1(ShiftToReg, IDEX_SLL, IDEX_SRL);

// MEM

wire [15:0] MemOut, AluOrMem;

wire NotZero, ne, eq, PCsrc;

reg MEMWB_RegWrite, MEMWB_MemToReg, MEMWB_ShiftToReg;

reg [15:0] DataMem[0:511];

reg [15:0] MEMWB_MemOut, MEMWB_ALUout, MEMWB_Shifted;

reg [15:0] MEMWB_InstrReg;

reg [1:0] MEMWB_WriteReg;

not g2(NotZero, EXMEM_Zero);

and g3(ne, EXMEM_BNE, NotZero),

g4(eq, EXMEM_BEQ, EXMEM_Zero);

or  g5(PCsrc, ne, eq);

assign MemOut = DataMem[EXMEM_ALUout];

always @(negedge clock) begin

if(EXMEM_MemWrite)

DataMem[EXMEM_ALUout] <= EXMEM_ReadData2;

end

// WB

mux_32_to_16 mux5(AluOrMem, MEMWB_ALUout, MEMWB_MemOut, MEMWB_MemToReg);

mux_32_to_16 mux6(WriteData, AluOrMem, MEMWB_Shifted, MEMWB_ShiftToReg);

initial begin

// Initialize registers

PC = 0;

IFID_InstrReg = 0;

IDEX_InstrReg = 0;

IDEX_RegDst = 0;

IDEX_BEQ = 0;

IDEX_BNE = 0;

IDEX_MemToReg = 0;

IDEX_MemWrite = 0;

IDEX_ALUsrc = 0;

IDEX_RegWrite = 0;

IDEX_SLL = 0;

IDEX_SRL = 0;

IDEX_shamt = 0;

IDEX_ALUctl = 3'b000;

IDEX_ReadData1 = 0;

IDEX_ReadData2 = 0;

IDEX_SignExtend = 0;

IDEX_rd = 0;

IDEX_rt = 0;

EXMEM_InstrReg = 0;

EXMEM_BEQ = 0;

EXMEM_BNE = 0;

EXMEM_MemToReg = 0;

EXMEM_MemWrite = 0;

EXMEM_RegWrite = 0;

EXMEM_ShiftToReg = 0;

EXMEM_ALUout = 0;

EXMEM_ReadData2 = 0;

EXMEM_SignExtend = 0;

EXMEM_Shifted = 0;

EXMEM_WriteReg = 0;

EXMEM_Zero = 0;

MEMWB_MemToReg = 0;

MEMWB_RegWrite = 0;

MEMWB_ShiftToReg = 0;

MEMWB_ALUout = 0;

MEMWB_InstrReg = 0;

MEMWB_MemOut = 0;

MEMWB_Shifted = 0;

MEMWB_WriteReg = 0;

end

always @(negedge clock) begin

// Stage 1 -- IF

IFID_InstrReg <= InstrMem[PC];

PC <= NextPC;

// Stage 2 -- ID

IDEX_InstrReg <= IFID_InstrReg;

{IDEX_RegDst, IDEX_BEQ, IDEX_BNE, IDEX_MemToReg, IDEX_MemWrite, IDEX_ALUsrc, IDEX_RegWrite, IDEX_SLL, IDEX_SRL, IDEX_ALUctl} <= control;

IDEX_ReadData1 <= ReadData1;

IDEX_ReadData2 <= ReadData2;

IDEX_SignExtend <= SignExtend;

IDEX_rt <= IFID_InstrReg[9:8];

IDEX_rd <= IFID_InstrReg[7:6];

IDEX_shamt <= IFID_InstrReg[5:2];

// Stage 3 -- EX

EXMEM_InstrReg <= IDEX_InstrReg;

EXMEM_RegWrite <= IDEX_RegWrite;

EXMEM_MemToReg <= IDEX_MemToReg;

EXMEM_ShiftToReg <= ShiftToReg;

EXMEM_BEQ <= IDEX_BEQ;

EXMEM_BNE <= IDEX_BNE;

EXMEM_MemWrite <= IDEX_MemWrite;

EXMEM_Zero <= Zero;

EXMEM_ALUout <= ALUout;

EXMEM_ReadData2 <= IDEX_ReadData2;

EXMEM_WriteReg <= WriteReg;

EXMEM_SignExtend <= IDEX_SignExtend;

EXMEM_Shifted <= Shifted;

// Stage 4 -- MEM

MEMWB_InstrReg <= EXMEM_InstrReg;

MEMWB_RegWrite <= EXMEM_RegWrite;

MEMWB_MemToReg <= EXMEM_MemToReg;

MEMWB_ShiftToReg <= EXMEM_ShiftToReg;

MEMWB_MemOut <= MemOut;

MEMWB_ALUout <= EXMEM_ALUout;

MEMWB_WriteReg <= EXMEM_WriteReg;

MEMWB_Shifted <= EXMEM_Shifted;

end

endmodule

## module main_control(Op,Control);

input [3:0] Op;

output reg [11:0] Control;

always @(Op) case (Op)

// {RegDst, BEQ, BNE, MemtoReg, MemWrite, ALUSrc, RegWrite, SLL, SRL, ALUctl[2], ALUctl[1], ALUctl[0]}

4'b0000: Control <= 12'b100000100010; // add

4'b0001: Control <= 12'b100000100110; // sub

4'b0010: Control <= 12'b100000100000; // and

4'b0011: Control <= 12'b100000100001; // or

4'b0100: Control <= 12'b000001100010; // addi

4'b0101: Control <= 12'b000111100010; // lw

4'b0110: Control <= 12'b000011000010; // sw

4'b0111: Control <= 12'b100000100111; // slt

4'b1000: Control <= 12'b010000000110; // beq

4'b1001: Control <= 12'b001000000110; // bne

4'b1010: Control <= 12'b100000110000; // sll

4'b1011: Control <= 12'b100000101000; // srl

endcase

endmodule

## module test ();

reg clock;

wire [15:0] PC, IFID_InstrReg, IDEX_InstrReg, EXMEM_InstrReg, MEMWB_InstrReg, WriteData;

CPU test_cpu(clock, PC, IFID_InstrReg, IDEX_InstrReg, EXMEM_InstrReg, MEMWB_InstrReg, WriteData);

always #1 clock = ~clock;

initial begin

$display ("time PC  IFID_IR  IDEX_IR  EXMEM_IR MEMWB_IR WD");

$monitor ("%2d  %3d  %h %h %h %h %h", $time, PC, IFID_InstrReg, IDEX_InstrReg, EXMEM_InstrReg, MEMWB_InstrReg, WriteData);

clock = 1;

#80 $finish;

end

endmodule

# Testing & Results

## Testing Code

### input4.hex

5100	// lw   $1, 0($0)  ==> 0101000100000000

5201	// lw   $2, 1($0)  ==> 0101001000000001

0000	// nop             ==> 0000000000000000

0000	// nop             ==> 0000000000000000

0000	// nop             ==> 0000000000000000

76c0	// slt  $3, $1, $2 ==> 0111011011000000

0000	// nop             ==> 0000000000000000

0000	// nop             ==> 0000000000000000

0000	// nop             ==> 0000000000000000

8c17	// beq  $3, $0, 23 ==> 1000110000010111

0000	// nop             ==> 0000000000000000

0000	// nop             ==> 0000000000000000

0000	// nop             ==> 0000000000000000

6101	// sw   $1, 1($0)  ==> 0110000100000001

6200	// sw   $2, 0($0)  ==> 0110001000000000

0000	// nop             ==> 0000000000000000

0000	// nop             ==> 0000000000000000

0000	// nop             ==> 0000000000000000

5100	// lw   $1, 0($0)  ==> 0101000100000000

5201	// lw   $2, 1($0)  ==> 0101001000000001

0000	// nop             ==> 0000000000000000

0000	// nop             ==> 0000000000000000

0000	// nop             ==> 0000000000000000

1640	// sub  $1, $1, $2 ==> 0001011001000000

0000	// nop             ==> 0000000000000000

0000	// nop             ==> 0000000000000000

0000	// nop             ==> 0000000000000000

a1a0	// sll $2, $1, 8   ==> 1010000110100000

0000	// nop             ==> 0000000000000000

0000	// nop             ==> 0000000000000000

0000	// nop             ==> 0000000000000000

b2a0	// srl $2, $2, 3   ==> 1011001010100000

## Testing Results

### Screenshot

### Plaintext

reishka@behemoth-ubu:~/cs385-finalProject$ iverilog -o test_cpu_4a -f modules4a.txt

reishka@behemoth-ubu:~/cs385-finalProject$ vvp test_cpu_4a

WARNING: cpu4a.v:21: $readmemh(input4.hex): Not enough words in the file for the requested range [0:511].

time PC  IFID_IR  IDEX_IR  EXMEM_IR  MEMWB_IR  WD

0    0  0000     0000     0000      0000      0000

1    1  5100     0000     0000      0000      0000

3    2  5201     5100     0000      0000      0000

5    3  0000     5201     5100      0000      0000

7    4  0000     0000     5201      5100      0400

9    5  0000     0000     0000      5201      0fad

11    6  76c0     0000     0000      0000      0000

13    7  0000     76c0     0000      0000      0000

15    8  0000     0000     76c0      0000      0000

17    9  0000     0000     0000      76c0      0001

19   10  8c17     0000     0000      0000      0000

21   11  0000     8c17     0000      0000      0000

23   12  0000     0000     8c17      0000      0000

25   13  0000     0000     0000      8c17      0001

27   14  6101     0000     0000      0000      0000

29   15  6200     6101     0000      0000      0000

31   16  0000     6200     6101      0000      0000

33   17  0000     0000     6200      6101      0001

35   18  0000     0000     0000      6200      0000

37   19  5100     0000     0000      0000      0000

39   20  5201     5100     0000      0000      0000

41   21  0000     5201     5100      0000      0000

43   22  0000     0000     5201      5100      0fad

45   23  0000     0000     0000      5201      0400

47   24  1640     0000     0000      0000      0000

49   25  0000     1640     0000      0000      0000

51   26  0000     0000     1640      0000      0000

53   27  0000     0000     0000      1640      0bad

55   28  a1a0     0000     0000      0000      0000

57   29  0000     a1a0     0000      0000      0000

59   30  0000     0000     a1a0      0000      0000

61   31  0000     0000     0000      a1a0      ad00

63   32  b2a0     0000     0000      0000      0000

65   33  xxxx     b2a0     0000      0000      0000

67   34  xxxx     xxxx     b2a0      0000      0000

69   35  xxxx     xxxx     xxxx      b2a0      00ad

71   36  xxxx     xxxx     xxxx      xxxx      xxxx

73   37  xxxx     xxxx     xxxx      xxxx      xxxx

75   38  xxxx     xxxx     xxxx      xxxx      xxxx

77   39  xxxx     xxxx     xxxx      xxxx      xxxx

79   40  xxxx     xxxx     xxxx      xxxx      xxxx

reishka@behemoth-ubu:~/cs385-finalProject$