# CS 385 – FINAL REPORT

RACHAEL GRABOWSKI & MICHAEL LANG

# INSTRUCTION SET

| Name | Syntax | Meaning | Format | Opcode | Encoding |
| --- | --- | --- | --- | --- | --- |
| Add | add $d, $s, $t | $d $s + $t | R | 0000 | 0000 sstt dd-- ---- |
| Subtract | sub $d, $s, $t | $d $s - $t | R | 0001 | 0001 sstt dd-- ---- |
| And | and $d, $s, $t | $d $s & $t | R | 0010 | 0010 sstt dd-- ---- |
| Or | or $d, $s, $t | $d $s ¦ $t | R | 0011 | 0011 sstt dd-- ---- |
| Add Immediate | addi $t, $s, C | $t $s + C | I | 0100 | 0100 sstt CCCC CCCC |
| Load word | lw $t, C($s) | $t DataMem[$s+C] | I | 0101 | 0101 sstt CCCC CCCC |
| Store word | sw $t, C($s) | DataMem[$s+C] $t | I | 0110 | 0110 sstt CCCC CCCC |
| Set on less than | slt $d, $s, $t | $d = ($s < $t) | R | 0111 | 0111 sstt dd-- ---- |
| Branch on equal | beq $s, $t, C | if ($s == $t) goto C | I | 1000 | 1000 sstt CCCC CCCC |
| Branch on not equal | bne $s, $t, C | if ($s != $t) goto C | I | 1001 | 1001 sstt CCCC CCCC |
| Shift left logical | sll $d, $t, shamt | $d $t << shamt | R | 1010 | 1010 --tt ddaa aa-- |
| Shift right logical | srl $d, $t, shamt | $d $t >> shamt | R | 1011 | 1011 --tt ddaa aa-- |
| Jump | j C | PC C | I | 1100 | 1100 ---- CCCC CCCC |

Our implementation of a 16-bit mini pipelined MIPS machine (using gate modeling and behavioral modeling in Verilog) includes the following instructions: add, sub, and, or, addi, lw, sw, slt, beq, bne, sll, srl, j (see the table above for corresponding OP codes).

| R-Format |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OP | RS | RT | RD | shamt | Unused |
| 4 | 2 | 2 | 2 | 4 | 2 |

| I-Format |  |  |  |
| --- | --- | --- | --- |
| OP | RS | RT | Address/Value |
| 4 | 2 | 2 | 8 |

# Major Component Descriptions

## Instruction Memory

The instruction memory is represented by our input.hex file (see “input.hex” below)

## ALU

The ALU is a 16-bit ALU comprised of four 4-bit ALUs. It is capable of detecting overflow and zero.

## Register File

Register r0 is static 0 (it cannot change or be changed). The register file is 4-word by 16-bits-per-word.

## Control Unit

The mainCtrl unit takes IFID_InstrMem[15:12] as input, and outputs a control signal to the IDEX register. The control options are: RegDst, BEQ, BNE, MemToReg, MemWrite, ALUsrc, RegWrite, SLL, SRL, and ALUctl.

## input.hex

The input.hex file is how we loaded our code into our program, instead of hard-coding our testing instructions. This made it very easy to swap between different sets of instructions for testing at various points in the project. MIPS ASM instructions were converted into binary, and those binary instructions were then converted into hex instructions. Those hex instructions were then read by the CPU module into instruction memory.

# DIAGRAM

Diagram is best viewed at 250% or larger

# VERILOG SOURCE CODE

## module alu_1_bit(result, c_out, a, b, less, c_in, op);

input a, b;

input less;

input c_in;

input [2:0] op;

output result, c_out;

wire m, n, p, q, r;

not g1(m, b);

mux_2_to_1  mux1(n, b, m, op[2]);

and g2(p, a, n);

or  g3(q, a, n);

full_adder  fa(r, c_out, a, n, c_in);

mux_4_to_1  mux2(result, p, q, r, less, op[1:0]);

endmodule

## module alu_1_bit_msb(result, overflow, set, c_out, a, b, less, c_in, op);

input a, b;

input less;

input c_in;

input [2:0] op;

output result, overflow, set, c_out;

wire m, n, p, q;

not g1(m, b);

mux_2_to_1  mux1(n, b, m, op[2]);

and g2(p, a, n);

or  g3(q, a, n);

full_adder  fa(set, c_out, a, n, c_in);

mux_4_to_1  mux2(result, p, q, set, less, op[1:0]);

xor g4(overflow, c_out, c_in);

endmodule

## module alu_4_bit(result, zero, c_out, a, b, less, c_in, op);

input [3:0] a;

input [3:0] b;

input less;

input c_in;

input [2:0] op;

output [3:0] result;

output zero, c_out;

wire p, q, r, s, t;

alu_1_bit alu_0(result[0], p, a[0], b[0], less, c_in, op),

alu_1(result[1], q, a[1], b[1], 0, p, op),

alu_2(result[2], r, a[2], b[2], 0, q, op),

alu_3(result[3], c_out, a[3], b[3], 0, r, op);

or  g1(s, result[0], result[1]),

g2(t, result[2], result[3]);

nor g3(zero, s, t);

endmodule

## module alu_4_bit_last(result, overflow, set, zero, c_out, a, b, c_in, op);

input [3:0] a;

input [3:0] b;

input c_in;

input [2:0] op;

output [3:0] result;

output overflow, set, zero, c_out;

wire p, q, r, s, t;

alu_1_bit alu_0(result[0], p, a[0], b[0], 0, c_in, op),

alu_1(result[1], q, a[1], b[1], 0, p, op),

alu_2(result[2], r, a[2], b[2], 0, q, op);

alu_1_bit_msb alu_3(result[3], overflow, set, c_out, a[3], b[3], 0, r, op);

or  g1(s, result[0], result[1]),

g2(t, result[2], result[3]);

nor g3(zero, s, t);

endmodule

## module alu_16_bit(result, overflow, zero, c_out, a, b, op);

input [15:0] a;

input [15:0] b;

input [2:0] op;

output [15:0] result;

output overflow, zero, c_out;

wire p, q, r, s, t, u, v, w, x;

wire less;

alu_4_bit alu_0(result[3:0], s, p, a[3:0], b[3:0], less, op[2], op),

alu_1(result[7:4], t, q, a[7:4], b[7:4], 0, p, op),

alu_2(result[11:8], u, r, a[11:8], b[11:8], 0, q, op);

alu_4_bit_last alu_3(result[15:12], overflow, less, v, c_out, a[15:12], b[15:12], r, op);

and  g1(w, s, t),

g2(x, u, v),

g3(zero, w, x);

endmodule

## module d_flip_flop(D,CLK,Q);

input D,CLK;

output Q;

wire CLK1, Y;

not  not1 (CLK1,CLK);

d_latch D1(D,CLK, Y),

D2(Y,CLK1,Q);

endmodule

## module d_latch(D,C,Q);

input D,C;

output Q;

wire x,y,D1,Q1;

nand nand1 (x,D, C),

nand2 (y,D1,C),

nand3 (Q,x,Q1),

nand4 (Q1,y,Q);

not  not1  (D1,D);

endmodule

## module decoder_2_to_4(out, sel);

input [1:0] sel;

output [3:0] out;

wire not_sel0, not_sel1;

not n0(not_sel0, sel[0]),

n1(not_sel1, sel[1]);

and a0(out[0], not_sel0, not_sel1),

a1(out[1], sel[0], not_sel1),

a2(out[2], not_sel0, sel[1]),

a3(out[3], sel[0], sel[1]);

endmodule

## module half_adder(sum, c_out, a, b);

input a, b;

output sum, c_out;

xor g1 (sum, a, b);

and g2 (c_out, a, b);

endmodule

## module full_adder(sum, c_out, a, b, c_in);

input a, b, c_in;

output sum, c_out;

wire p, q, r;

half_adder ha1(p, q, a, b),

ha2(sum, r, p, c_in);

or g1(c_out, r, q);

endmodule

## module mux_2_to_1(x, a, b, sel);

input a, b, sel;

output x;

wire p, q, r;

not g1 (p, sel);

and g2 (q, a, p),

g3 (r, b, sel);

or  g4 (x, q, r);

endmodule

## module mux_4_to_1(x, a, b, c, d, sel);

input a, b, c, d;

input [1:0] sel;

output x;

wire p, q;

mux_2_to_1 mux1(p, a, b, sel[0]),

mux2(q, c, d, sel[0]),

mux3(x, p, q, sel[1]);

endmodule

## module mux_4_to_2(x, a, b, sel);

input [1:0] a, b;

input sel;

output [1:0] x;

mux_2_to_1 mux1(x[0], a[0], b[0], sel),

mux2(x[1], a[1], b[1], sel);

endmodule

## module mux_32_to_16(x, a, b, sel);

input [15:0] a, b;

input sel;

output [15:0] x;

mux_2_to_1 m0(x[0], a[0], b[0], sel),

m1(x[1], a[1], b[1], sel),

m2(x[2], a[2], b[2], sel),

m3(x[3], a[3], b[3], sel),

m4(x[4], a[4], b[4], sel),

m5(x[5], a[5], b[5], sel),

m6(x[6], a[6], b[6], sel),

m7(x[7], a[7], b[7], sel),

m8(x[8], a[8], b[8], sel),

m9(x[9], a[9], b[9], sel),

m10(x[10], a[10], b[10], sel),

m11(x[11], a[11], b[11], sel),

m12(x[12], a[12], b[12], sel),

m13(x[13], a[13], b[13], sel),

m14(x[14], a[14], b[14], sel),

m15(x[15], a[15], b[15], sel);

endmodule

## module mux_64_to_16(x, a, b, c, d, sel);

input [15:0] a, b, c, d;

input [1:0] sel;

output [15:0] x;

mux_4_to_1 m0(x[0], a[0], b[0], c[0], d[0], sel),

m1(x[1], a[1], b[1], c[1], d[1], sel),

m2(x[2], a[2], b[2], c[2], d[2], sel),

m3(x[3], a[3], b[3], c[3], d[3], sel),

m4(x[4], a[4], b[4], c[4], d[4], sel),

m5(x[5], a[5], b[5], c[5], d[5], sel),

m6(x[6], a[6], b[6], c[6], d[6], sel),

m7(x[7], a[7], b[7], c[7], d[7], sel),

m8(x[8], a[8], b[8], c[8], d[8], sel),

m9(x[9], a[9], b[9], c[9], d[9], sel),

m10(x[10], a[10], b[10], c[10], d[10], sel),

m11(x[11], a[11], b[11], c[11], d[11], sel),

m12(x[12], a[12], b[12], c[12], d[12], sel),

m13(x[13], a[13], b[13], c[13], d[13], sel),

m14(x[14], a[14], b[14], c[14], d[14], sel),

m15(x[15], a[15], b[15], c[15], d[15], sel);

endmodule

module register_16_bit(out, in, clk);

input [15:0] in;

input clk;

output [15:0] out;

d_flip_flop d0(in[0], clk, out[0]),

d1(in[1], clk, out[1]),

d2(in[2], clk, out[2]),

d3(in[3], clk, out[3]),

d4(in[4], clk, out[4]),

d5(in[5], clk, out[5]),

d6(in[6], clk, out[6]),

d7(in[7], clk, out[7]),

d8(in[8], clk, out[8]),

d9(in[9], clk, out[9]),

d10(in[10], clk, out[10]),

d11(in[11], clk, out[11]),

d12(in[12], clk, out[12]),

d13(in[13], clk, out[13]),

d14(in[14], clk, out[14]),

d15(in[15], clk, out[15]);

endmodule

module register_file_4_by_16(read_data1, read_data2, read_reg1, read_reg2, write_reg, write_data, reg_write, clk);

input [1:0] read_reg1, read_reg2, write_reg;

input [15:0] write_data;

input reg_write, clk;

output [15:0] read_data1, read_data2;

wire [3:0] w, c;

wire p;

wire [15:0] q1, q2, q3;

decoder_2_to_4 dec(w, write_reg);

and g1(p, reg_write, clk),

g2(c[1], p, w[1]),

g3(c[2], p, w[2]),

g4(c[3], p, w[3]);

register_16_bit r1(q1, write_data, c[1]),

r2(q2, write_data, c[2]),

r3(q3, write_data, c[3]);

mux_64_to_16 m1(read_data1, 0, q1, q2, q3, read_reg1),

m2(read_data2, 0, q1, q2, q3, read_reg2);

endmodule

## module shift_left_logical(out, in, shift_amount);

input [15:0] in;

input [3:0] shift_amount;

output [15:0] out;

wire [15:0] p, q, r;

mux_32_to_16 mux1(p, in[15:0], {in[7:0],8'b0}, shift_amount[3]),

mux2(q, p, {p[11:0],4'b0}, shift_amount[2]),

mux3(r, q, {q[13:0],2'b0}, shift_amount[1]),

mux4(out, r, {r[14:0],1'b0}, shift_amount[0]);

endmodule

## module shift_right_logical(out, in, shift_amount);

input [15:0] in;

input [3:0] shift_amount;

output [15:0] out;

wire [15:0] p, q, r;

mux_32_to_16 mux1(p, in[15:0], {8'b0,in[15:8]}, shift_amount[3]),

mux2(q, p, {4'b0,p[15:4]}, shift_amount[2]),

mux3(r, q, {2'b0,q[15:2]}, shift_amount[1]),

mux4(out, r, {1'b0,r[15:1]}, shift_amount[0]);

endmodule

## module CPU (clock, PC, IFID_InstrReg, IDEX_InstrReg, EXMEM_InstrReg, MEMWB_InstrReg, WriteData);

input clock;

output [15:0] PC, WriteData, IFID_InstrReg, IDEX_InstrReg, EXMEM_InstrReg, MEMWB_InstrReg;

// IF

wire [15:0] NextPC, PCplus1;

reg [15:0] PC, IFID_InstrReg;

reg [15:0] InstrMem[0:511];

alu_16_bit fetch(PCplus1, unused1, unused2, unused3, PC, 1, 3'b010);

mux_32_to_16 mux1(NextPC, PCplus1, EXMEM_SignExtend, PCsrc);

// Read hex data from text file into InstrMem

initial begin

$readmemh("input4.hex", InstrMem);

DataMem[0] = 16'h400;

DataMem[1] = 16'hfad;

end

// ID

reg [15:0] IDEX_InstrReg;

wire [11:0] control;

reg IDEX_RegDst, IDEX_BEQ, IDEX_BNE, IDEX_MemToReg, IDEX_MemWrite, IDEX_ALUsrc, IDEX_RegWrite, IDEX_SLL, IDEX_SRL;

reg [2:0] IDEX_ALUctl;

wire [15:0] ReadData1, ReadData2, SignExtend, WriteData;

reg [15:0] IDEX_ReadData1, IDEX_ReadData2, IDEX_SignExtend;

reg [1:0] IDEX_rt, IDEX_rd;

reg [3:0] IDEX_shamt;

register_file_4_by_16 regFile(ReadData1, ReadData2, IFID_InstrReg[11:10], IFID_InstrReg[9:8], MEMWB_WriteReg, WriteData, MEMWB_RegWrite, clock);

main_control mainCtrl(IFID_InstrReg[15:12], control);

assign SignExtend = {{8{IFID_InstrReg[7]}},IFID_InstrReg[7:0]}; // sign extension

// EX

wire [15:0] B, ALUout, ShiftLOut, ShiftROut, Shifted;

wire [1:0] WriteReg;

wire Zero, ShiftToReg;

reg EXMEM_RegWrite, EXMEM_MemToReg;

reg EXMEM_MemWrite, EXMEM_BEQ, EXMEM_BNE, EXMEM_ShiftToReg;

reg EXMEM_Zero;

reg [15:0] EXMEM_SignExtend, EXMEM_ALUout, EXMEM_ReadData2, EXMEM_Shifted;

reg [15:0] EXMEM_InstrReg;

reg [1:0] EXMEM_WriteReg;

alu_16_bit ex(ALUout, unused4, Zero, unused5, IDEX_ReadData1, B, IDEX_ALUctl);

mux_4_to_2 mux2(WriteReg, IDEX_rt, IDEX_rd, IDEX_RegDst);

mux_32_to_16 mux3(B, IDEX_ReadData2, IDEX_SignExtend, IDEX_ALUsrc);

shift_left_logical sll(ShiftLOut, IDEX_ReadData2, IDEX_shamt);

shift_right_logical srl(ShiftROut, IDEX_ReadData2, IDEX_shamt);

mux_32_to_16 mux4(Shifted, ShiftLOut, ShiftROut, IDEX_SRL);

or g1(ShiftToReg, IDEX_SLL, IDEX_SRL);

// MEM

wire [15:0] MemOut, AluOrMem;

wire NotZero, ne, eq, PCsrc;

reg MEMWB_RegWrite, MEMWB_MemToReg, MEMWB_ShiftToReg;

reg [15:0] DataMem[0:511];

reg [15:0] MEMWB_MemOut, MEMWB_ALUout, MEMWB_Shifted;

reg [15:0] MEMWB_InstrReg;

reg [1:0] MEMWB_WriteReg;

not g2(NotZero, EXMEM_Zero);

and g3(ne, EXMEM_BNE, NotZero),

g4(eq, EXMEM_BEQ, EXMEM_Zero);

or  g5(PCsrc, ne, eq);

assign MemOut = DataMem[EXMEM_ALUout];

always @(negedge clock) begin

if(EXMEM_MemWrite)

DataMem[EXMEM_ALUout] <= EXMEM_ReadData2;

end

// WB

mux_32_to_16 mux5(AluOrMem, MEMWB_ALUout, MEMWB_MemOut, MEMWB_MemToReg);

mux_32_to_16 mux6(WriteData, AluOrMem, MEMWB_Shifted, MEMWB_ShiftToReg);

initial begin

// Initialize registers

PC = 0;

IFID_InstrReg = 0;

IDEX_InstrReg = 0;

IDEX_RegDst = 0;

IDEX_BEQ = 0;

IDEX_BNE = 0;

IDEX_MemToReg = 0;

IDEX_MemWrite = 0;

IDEX_ALUsrc = 0;

IDEX_RegWrite = 0;

IDEX_SLL = 0;

IDEX_SRL = 0;

IDEX_shamt = 0;

IDEX_ALUctl = 3'b000;

IDEX_ReadData1 = 0;

IDEX_ReadData2 = 0;

IDEX_SignExtend = 0;

IDEX_rd = 0;

IDEX_rt = 0;

EXMEM_InstrReg = 0;

EXMEM_BEQ = 0;

EXMEM_BNE = 0;

EXMEM_MemToReg = 0;

EXMEM_MemWrite = 0;

EXMEM_RegWrite = 0;

EXMEM_ShiftToReg = 0;

EXMEM_ALUout = 0;

EXMEM_ReadData2 = 0;

EXMEM_SignExtend = 0;

EXMEM_Shifted = 0;

EXMEM_WriteReg = 0;

EXMEM_Zero = 0;

MEMWB_MemToReg = 0;

MEMWB_RegWrite = 0;

MEMWB_ShiftToReg = 0;

MEMWB_ALUout = 0;

MEMWB_InstrReg = 0;

MEMWB_MemOut = 0;

MEMWB_Shifted = 0;

MEMWB_WriteReg = 0;

end

always @(negedge clock) begin

// Stage 1 -- IF

IFID_InstrReg <= InstrMem[PC];

PC <= NextPC;

// Stage 2 -- ID

IDEX_InstrReg <= IFID_InstrReg;

{IDEX_RegDst, IDEX_BEQ, IDEX_BNE, IDEX_MemToReg, IDEX_MemWrite, IDEX_ALUsrc, IDEX_RegWrite, IDEX_SLL, IDEX_SRL, IDEX_ALUctl} <= control;

IDEX_ReadData1 <= ReadData1;

IDEX_ReadData2 <= ReadData2;

IDEX_SignExtend <= SignExtend;

IDEX_rt <= IFID_InstrReg[9:8];

IDEX_rd <= IFID_InstrReg[7:6];

IDEX_shamt <= IFID_InstrReg[5:2];

// Stage 3 -- EX

EXMEM_InstrReg <= IDEX_InstrReg;

EXMEM_RegWrite <= IDEX_RegWrite;

EXMEM_MemToReg <= IDEX_MemToReg;

EXMEM_ShiftToReg <= ShiftToReg;

EXMEM_BEQ <= IDEX_BEQ;

EXMEM_BNE <= IDEX_BNE;

EXMEM_MemWrite <= IDEX_MemWrite;

EXMEM_Zero <= Zero;

EXMEM_ALUout <= ALUout;

EXMEM_ReadData2 <= IDEX_ReadData2;

EXMEM_WriteReg <= WriteReg;

EXMEM_SignExtend <= IDEX_SignExtend;

EXMEM_Shifted <= Shifted;

// Stage 4 -- MEM

MEMWB_InstrReg <= EXMEM_InstrReg;

MEMWB_RegWrite <= EXMEM_RegWrite;

MEMWB_MemToReg <= EXMEM_MemToReg;

MEMWB_ShiftToReg <= EXMEM_ShiftToReg;

MEMWB_MemOut <= MemOut;

MEMWB_ALUout <= EXMEM_ALUout;

MEMWB_WriteReg <= EXMEM_WriteReg;

MEMWB_Shifted <= EXMEM_Shifted;

end

endmodule

## module main_control(Op,Control);

input [3:0] Op;

output reg [11:0] Control;

always @(Op) case (Op)

// {RegDst, BEQ, BNE, MemtoReg, MemWrite, ALUSrc, RegWrite, SLL, SRL, ALUctl[2], ALUctl[1], ALUctl[0]}

4'b0000: Control <= 12'b100000100010; // add

4'b0001: Control <= 12'b100000100110; // sub

4'b0010: Control <= 12'b100000100000; // and

4'b0011: Control <= 12'b100000100001; // or

4'b0100: Control <= 12'b000001100010; // addi

4'b0101: Control <= 12'b000111100010; // lw

4'b0110: Control <= 12'b000011000010; // sw

4'b0111: Control <= 12'b100000100111; // slt

4'b1000: Control <= 12'b010000000110; // beq

4'b1001: Control <= 12'b001000000110; // bne

4'b1010: Control <= 12'b100000110000; // sll

4'b1011: Control <= 12'b100000101000; // srl

4'b1100: Control <= 12'b011000000110; // j

endcase

endmodule

## module test ();

reg clock;

wire [15:0] PC, IFID_InstrReg, IDEX_InstrReg, EXMEM_InstrReg, MEMWB_InstrReg, WriteData;

CPU test_cpu(clock, PC, IFID_InstrReg, IDEX_InstrReg, EXMEM_InstrReg, MEMWB_InstrReg, WriteData);

always #1 clock = ~clock;

initial begin

$display ("time PC  IFID_IR  IDEX_IR  EXMEM_IR MEMWB_IR WD");

$monitor ("%2d  %3d  %h %h %h %h %h", $time, PC, IFID_InstrReg, IDEX_InstrReg, EXMEM_InstrReg, MEMWB_InstrReg, WriteData);

clock = 1;

#163 $finish;

end

endmodule

# Testing & Results

## Testing Code

The test code was run twice – once with nops and once without. Here, the hex file with nops is included. However, the screenshot and plaintext output of both runs are below.

### input4.hex

5100    // lw $1, 0($0)   ==> 0101 0001 0000 0000	// $1 <- x

5201    // lw $2, 1($0)   ==> 0101 0010 0000 0001	// $2 <- y

4300    // addi $3, $0, 0 ==> 0100 0011 0000 0000	// $3 <- 0

0000    // nop            ==> 0000 0000 0000 0000  // nop

0000    // nop            ==> 0000 0000 0000 0000  // nop

8827    // beq $2, $0, 39 ==> 1000 1000 0010 0111  // if y == 0 goto 39

0000    // nop            ==> 0000 0000 0000 0000  // nop

0000    // nop            ==> 0000 0000 0000 0000  // nop

0000    // nop            ==> 0000 0000 0000 0000  // nop

6302    // sw $3, 2($0)   ==> 0110 0011 0000 0010  // a <- $3

0000    // nop            ==> 0000 0000 0000 0000  // nop

0000    // nop            ==> 0000 0000 0000 0000  // nop

0000    // nop            ==> 0000 0000 0000 0000  // nop

4301    // addi $3, $0, 1 ==> 0100 0011 0000 0001  // $3 <- 1

0000    // nop            ==> 0000 0000 0000 0000  // nop

0000    // nop            ==> 0000 0000 0000 0000  // nop

0000    // nop            ==> 0000 0000 0000 0000  // nop

2bc0    // and $3, $2, $3 ==> 0010 1011 1100 0000  // $3 <- y && 1

0000    // nop            ==> 0000 0000 0000 0000  // nop

0000    // nop            ==> 0000 0000 0000 0000  // nop

0000    // nop            ==> 0000 0000 0000 0000  // nop

8c1e    // beq $3, $0, 30 ==> 1000 1100 0001 1110  // if $3 == 0 goto 30

0000    // nop            ==> 0000 0000 0000 0000  // nop

0000    // nop            ==> 0000 0000 0000 0000  // nop

0000    // nop            ==> 0000 0000 0000 0000  // nop

5302    // lw $3, 2($0)   ==> 0101 0011 0000 0010  // $3 <- a

0000    // nop            ==> 0000 0000 0000 0000  // nop

0000    // nop            ==> 0000 0000 0000 0000  // nop

0000    // nop            ==> 0000 0000 0000 0000  // nop

0dc0    // add $3, $3, $1 ==> 0000 1101 1100 0000  // $3 <- a + x

a144    // sll $1, $1, 1  ==> 1010 0001 0100 0100  // x *= 2

b286    // srl $2, $2, 1  ==> 1011 0010 1000 0100  // y //= 2

0000    // nop            ==> 0000 0000 0000 0000  // nop

0000    // nop            ==> 0000 0000 0000 0000  // nop

0000    // nop            ==> 0000 0000 0000 0000  // nop

c005    // j 5            ==> 1100 0000 0000 0101  // goto 5

0000    // nop            ==> 0000 0000 0000 0000  // nop

0000    // nop            ==> 0000 0000 0000 0000  // nop

0000    // nop            ==> 0000 0000 0000 0000  // nop

4f00    // addi $3, $3, 0 ==> 0100 1111 0000 0000  // a += 0

## Testing Results

### Screenshot (with NOP)

### Plaintext (with NOP)

reishka@behemoth-ubu:~/cs385-finalProject$ iverilog -o test_cpu_4a -f modules4a.txt

reishka@behemoth-ubu:~/cs385-finalProject$ vvp test_cpu_4a

WARNING: cpu4a.v:21: $readmemh(input4.hex): Not enough words in the file for the requested range [0:511].

time PC  IFID_IR  IDEX_IR  EXMEM_IR  MEMWB_IR  WD

0    0  0000     0000     0000      0000      0000

1    1  5100     0000     0000      0000      0000

3    2  5201     5100     0000      0000      0000

5    3  4300     5201     5100      0000      0000

7    4  0000     4300     5201      5100      1ea3

9    5  0000     0000     4300      5201      0003

11    6  8827     0000     0000      4300      0000

13    7  0000     8827     0000      0000      0000

15    8  0000     0000     8827      0000      0000

17    9  0000     0000     0000      8827      0003

19   10  6302     0000     0000      0000      0000

21   11  0000     6302     0000      0000      0000

23   12  0000     0000     6302      0000      0000

25   13  0000     0000     0000      6302      0002

27   14  4301     0000     0000      0000      0000

29   15  0000     4301     0000      0000      0000

31   16  0000     0000     4301      0000      0000

33   17  0000     0000     0000      4301      0001

35   18  2bc0     0000     0000      0000      0000

37   19  0000     2bc0     0000      0000      0000

39   20  0000     0000     2bc0      0000      0000

41   21  0000     0000     0000      2bc0      0001

43   22  8c1e     0000     0000      0000      0000

45   23  0000     8c1e     0000      0000      0000

47   24  0000     0000     8c1e      0000      0000

49   25  0000     0000     0000      8c1e      0001

51   26  5302     0000     0000      0000      0000

53   27  0000     5302     0000      0000      0000

55   28  0000     0000     5302      0000      0000

57   29  0000     0000     0000      5302      0000

59   30  0dc0     0000     0000      0000      0000

61   31  a144     0dc0     0000      0000      0000

63   32  b286     a144     0dc0      0000      0000

65   33  0000     b286     a144      0dc0      1ea3

67   34  0000     0000     b286      a144      3d46

69   35  0000     0000     0000      b286      0001

71   36  c005     0000     0000      0000      0000

73   37  0000     c005     0000      0000      0000

75   38  0000     0000     c005      0000      0000

77    5  0000     0000     0000      c005      0000

79    6  8827     0000     0000      0000      0000

81    7  0000     8827     0000      0000      0000

83    8  0000     0000     8827      0000      0000

85    9  0000     0000     0000      8827      0001

87   10  6302     0000     0000      0000      0000

89   11  0000     6302     0000      0000      0000

91   12  0000     0000     6302      0000      0000

93   13  0000     0000     0000      6302      0002

95   14  4301     0000     0000      0000      0000

97   15  0000     4301     0000      0000      0000

99   16  0000     0000     4301      0000      0000

101   17  0000     0000     0000      4301      0001

103   18  2bc0     0000     0000      0000      0000

105   19  0000     2bc0     0000      0000      0000

107   20  0000     0000     2bc0      0000      0000

109   21  0000     0000     0000      2bc0      0001

111   22  8c1e     0000     0000      0000      0000

113   23  0000     8c1e     0000      0000      0000

115   24  0000     0000     8c1e      0000      0000

117   25  0000     0000     0000      8c1e      0001

119   26  5302     0000     0000      0000      0000

121   27  0000     5302     0000      0000      0000

123   28  0000     0000     5302      0000      0000

125   29  0000     0000     0000      5302      1ea3

127   30  0dc0     0000     0000      0000      0000

129   31  a144     0dc0     0000      0000      0000

131   32  b286     a144     0dc0      0000      0000

133   33  0000     b286     a144      0dc0      5be9

135   34  0000     0000     b286      a144      7a8c

137   35  0000     0000     0000      b286      0000

139   36  c005     0000     0000      0000      0000

141   37  0000     c005     0000      0000      0000

143   38  0000     0000     c005      0000      0000

145    5  0000     0000     0000      c005      0000

147    6  8827     0000     0000      0000      0000

149    7  0000     8827     0000      0000      0000

151    8  0000     0000     8827      0000      0000

153   39  0000     0000     0000      8827      0000

155   40  4f00     0000     0000      0000      0000

157   41  xxxx     4f00     0000      0000      0000

159   42  xxxx     xxxx     4f00      0000      0000

161   43  xxxx     xxxx     xxxx      4f00      5be9

163   44  xxxx     xxxx     xxxx      xxxx      xxxx

reishka@behemoth-ubu:~/cs385-finalProject$

### Screenshot (without NOP)

### Plaintext (without NOP)

reishka@behemoth-ubu:~/cs385-finalProject$ iverilog -o test_cpu_4a -f modules4a.txt

reishka@behemoth-ubu:~/cs385-finalProject$ vvp test_cpu_4a

WARNING: cpu4a.v:21: $readmemh(input4.hex): Not enough words in the file for the requested range [0:511].

time PC  IFID_IR  IDEX_IR  EXMEM_IR  MEMWB_IR  WD

0    0  0000     0000     0000      0000      0000

1    1  5100     0000     0000      0000      0000

3    2  5201     5100     0000      0000      0000

5    3  4300     5201     5100      0000      0000

7    4  8827     4300     5201      5100      1ea3

9    5  6302     8827     4300      5201      0003

11    6  4301     6302     8827      4300      0000

13    X  2bc0     4301     6302      8827      xxxx

15    X  xxxx     2bc0     4301      6302      0002

17    X  xxxx     xxxx     2bc0      4301      0001

19    X  xxxx     xxxx     xxxx      2bc0      0000

21    X  xxxx     xxxx     xxxx      xxxx      XXXX

23    X  xxxx     xxxx     xxxx      xxxx      XXXX

25    X  xxxx     xxxx     xxxx      xxxx      XXXX

27    X  xxxx     xxxx     xxxx      xxxx      XXXX

29    X  xxxx     xxxx     xxxx      xxxx      XXXX

31    X  xxxx     xxxx     xxxx      xxxx      XXXX

33    X  xxxx     xxxx     xxxx      xxxx      XXXX

35    X  xxxx     xxxx     xxxx      xxxx      XXXX

37    x  xxxx     xxxx     xxxx      xxxx      XXXX

reishka@behemoth-ubu:~/cs385-finalProject$